01․ Which of the following memories uses one transistor and one capacitor as basic memory unit

- SRAM

- DRAM

- Both SRAM and DRAM

- None

02․ Which number system has a base of 16

- Decimal

- Octal

- Hexadecimal

- None

03․ The Radix of and octal no is

a)8	 	b)2 			 c)16			 d)10

04․ How many entries will be in the truth table of a 3 input NAND gate ?

- 3

- 6

- 8

- 9

05․ How many bits are required to store one BCD digit ?

- 1

- 2

- 3

- 4

- 06․ In binary number system the first digit (bit) from right to left is called as

- LSB, Least Significant Bit

- MSB, Most Significant Bit

- First Bit

- Last Bit

07․ In an SR latch built from NOR gates, which condition is not allowed

- S=0, R=0

- S=0, R=1

- S=1, R=0

- S=1, R=1

08․ A D-flip-flop is said to be transparent when

- the output is LOW

- the output is HIGH

- the output follows clock

- the output follow input

09․ Which of these sets of logic gates are designated as universal gates?

- NOR, NAND.

- XOR, NOR, NAND.

- OR, NOT, AND.

- NOR, NAND, XNOR.

10․ Which one of the following is not a vectored interrupt?

- TRAP.

- RST 7.5.

- RST 6.5.

- INTR.

.

Q.11 The NAND gate output will be low if the two inputs are
(A) 00
(B) 01
(C) 10
(D) 11

Ans: D

Q.12  What is the binary equivalent of the decimal number 368
(A) 101110000
(B) 110110000
(C) 111010000
(D) 111100000

Ans: A

Q13.The decimal equivalent of hex number 1A53 is
(A) 6793
(B) 6739
(C) 6973
(D) 6379

Ans: B

DIGITAL ELECTRONICS 100 IMPORTANT MCQ

Q.14  (734)8 = ( )16
(A)    C 1 D
(B) D C 1
(C) 1 C D
(D) 1 D C

Ans: D

Q.15  The hexadecimal number ‘A0’ has the decimal value equivalent to
(A) 80
(B) 256
(C) 100
(D) 160

Ans: D

Q.16  The output of a logic gate is 1 when all its inputs are at logic 0. the gate is either
(A) a NAND or an EX-OR
(B) an OR or an EX-NOR
(C) an AND or an EX-OR
(D) a NOR or an EX-NOR

Ans: D

Q.17 A ring counter consisting of five Flip-Flops will have
(A) 5 states
(B) 10 states
(C) 32 states
(D) Infinite states.

Ans: A

Q.18 The speed of conversion is maximum in
(A) Successive-approximation A/D converter.
(B) Parallel-comparative A/D converter.
(C) Counter ramp A/D converter.
(D) Dual-slope A/D converter.

Ans: B

Q.19 The 2’s complement of the number 1101101 is
(A) 0101110
(B) 0111110
(C) 0110010
(D) 0010011

Ans: D

Q.20 The correction to be applied in decimal adder to the generated sum is
(A) 00101
(B) 00110
(C) 01101
(D) 01010

Ans: B

Q.21 The gates required to build a half adder are
(A) EX-OR gate and NOR gate
(B) EX-OR gate and OR gate
(C) EX-OR gate and AND gate
(D) Four NAND gates.

Ans: C

Q.22 The code where all successive numbers differ from their preceding number by single bit is
(A) Binary code.
(B) BCD.
(C) Excess – 3.
(D) Gray.

Ans: D

Q.23 If the input to T-flipflop is 100 Hz signal, the final output of the three T-flipflops in cascade is
(A) 1000 Hz
(B) 500 Hz
(C) 333 Hz
(D) 12.5 Hz.

Ans: D

Q.25 -8 is equal to signed binary number
(A) 10001000
(B) 00001000
(C) 10000000
(D) 11000000

Ans: A

Q.27 The digital logic family which has the lowest propagation delay time is
(A) ECL
(B) TTL
(C) CMOS
(D) PMOS

Ans: A

Q.28 The device which changes from serial data to parallel data is
(A)  COUNTER
(B) MULTIPLEXER
(C) DEMULTIPLEXER
(D) FLIP-FLOP

Ans: C

Q.29 A device which converts BCD to Seven Segment is called
(A) Encoder
(B) Decoder
(C) Multiplexer
(D) Demultiplexer

Ans: B

Q.30 In a JK Flip-Flop, toggle means
(A)  Set Q = 1 and Q = 0.
(B) Set Q = 0 and Q = 1.
(C)  Change the output to the opposite state.
(D)  No change in output.

Ans: C

Q.31 The access time of ROM using bipolar transistors is about
(A) 1 sec
(B) 1 msec
(C) 1 µsec
(D) 1 nsec.

Ans: C

Q.32 The A/D converter whose conversion time is independent of the number of bits is
(A)   Dual slope
(B) Counter type
(C) Parallel conversion
(D) Successive approximation.

Ans:  C

Q.33 When signed numbers are used in binary arithmetic, then which one of the following notations would have unique representation for zero.
(A) Sign-magnitude.
(B) 1’s complement.
(C) 2’s complement.
(D) 9’s complement.

Ans: A

Q.38 A 4-bit synchronous counter uses flip-flops with propagation delay times of 15 ns each. The maximum possible time required for change of state will be
(A) 15 ns.
(B)  30 ns.
(C) 45 ns.
(D)  60 ns.

Ans: A

Q.39 Words having 8-bits are to be stored into computer memory. The number of lines required for writing into memory are
(A) 1.
(B) 2.
(C) 4.
(D) 8.

Ans: D

Q.40 The decimal equivalent of Binary number 11010 is
(A) 26.
(B) 36.
(C)  16.
(D) 23.

Ans: A

Q.41 1’s complement representation of decimal number of -17 by using 8 bit representation is
(A) 1110 1110
(B)  1101 1101
(C) 1100 1100
(D)  0001 0001

Ans: A

Q.42 The excess 3 code of decimal number 26 is
(A) 0100 1001
(B) 01011001
(C) 1000 1001
(D) 01001101

DIGITAL ELECTRONICS 100 IMPORTANT MCQ

Q.43 How many AND gates are required to realize Y = CD+EF+G
(A) 4
(B) 5
(C) 3
(D) 2

Ans: D

Q.44 How many flip flops are required to construct a decade counter
(A) 10
(B) 3
(C) 4
(D) 2

Ans: C

Q.45  The chief reason why digital computers use complemented subtraction is that it
(A) Simplifies the circuitry.
(B) Is a very simple process.
(C) Can handle negative numbers easily.
(D) Avoids direct subtraction

Ans: C

Q.46 In a positive logic system, logic state 1 corresponds to
(A) positive voltage
(B)  higher voltage level
(C) zero voltage level
(D)  lower voltage level

Ans: B

Ans: B

Q.47 The number 140 in octal is equivalent to
(A)  (96)10 .
(B)  (86)10 .
(C)  (90)10 .
(D)  none of these.

Ans: A

Q.48 The NOR gate output will be low if the two inputs are
(A) 00
(B) 01
(C) 10
(D) 11

Q.49 How many flip-flops are required to construct mod 30 counter
(A) 5
(B) 6
(C) 4
(D) 8

Ans: A

Q.50 How many address bits are required to represent a 32 K memory
(A) 10 bits.
(B)  12 bits.
(C) 14 bits.
(D)  16 bits.

Ans: D

Q.51 Shifting a register content to left by one bit position is equivalent to
(A) division by two.
(B) addition by two.
(C) multiplication by two.
(D) subtraction by two.

Ans: C

Q.52 For JK flip flop with J=1, K=0, the output after clock pulse will be
(A) 0.
(B) 1.
(C) high impedance.
(D)  no change.

Ans: B

Q.53 Convert decimal 153 to octal. Equivalent in octal will be
(A)  (231)8 .
(B)  (331)8 .
(C)  (431)8 .
(D)  none of these.

Ans: A

Q.54 The decimal equivalent of (1100)2 is
(A) 12
(B) 16
(C) 18
(D) 20

Ans: A

Q.55 The binary equivalent of (FA)16 is
(A) 1010 1111
(B) 1111 1010
(C) 10110011
(D) none of these

Ans: B

Q.56 The output of SR flip flop when S=1, R=0 is
(A) 1
(B) 0
(C) No change
(D) High impedance

Ans: A

Q.57 The number of flip flops contained in IC 7490 is
(A) 2.
(B) 3.
(C) 4.
(D) 10.

Ans: A

Q.58 How many two-input AND and OR gates are required to realize Y=CD+EF+G
(A) 2,2.
(B) 2,3.
(C) 3,3.
(D)  none of these.

Ans: A

Q.59 The excess-3 code of decimal 7 is represented by
(A) 1100.
(B) 1001.
(C) 1011.
(D) 1010.

Ans: D

Q.60 The result of adding hexadecimal number A6 to 3A is
(A) DD.
(B) E0.
(C) F0.
(D) EF.

Ans: B

Q.61 A universal logic gate is one, which can be used to generate any logic function. Which of the following is a universal logic gate?
(A) OR
(B) AND
(C) XOR
(D) NAND

Ans: D

Q.62 Karnaugh map is used for the purpose of
(A) Reducing the electronic circuits used.
(B) To map the given Boolean logic function.
(C) To minimize the terms in a Boolean expression.
(D) To maximize the terms of a given a Boolean expression.

Ans: C

Q.63 A full adder logic circuit will have
(A) Two inputs and one output.
(B) Three inputs and three outputs.
(C) Two inputs and two outputs.
(D) Three inputs and two outputs.

Ans: D

Q.64 The 2’s complement of the number 1101110 is
(A) 0010001.
(B) 0010001.
(C) 0010010.
(D) None.

Ans: C

Q.65 The decimal equivalent of Binary number 10101 is
(A) 21
(B) 31
(C) 26
(D) 28

Ans: A

Q.66 How many two input AND gates and two input OR gates are required to realize Y = BD+CE+AB
(A) 1, 1
(B)  4, 2
(C) 3, 2
(D)  2, 3

Ans: A

Q.67 Which of following are known as universal gates
(A) NAND & NOR.
(B)  AND & OR.
(C)  XOR & OR.
(D) None.

Ans: A

Q.68 Which of the following memories stores the most number of bits
(A) 64K ´ 8 memory.
(B) 1M 8 memory.
(C) 32M 8 memory.
(D) 64 ´ 6 memory.

Ans: C

Q.69 Which of following consume minimum power
(A) TTL.
(B) CMOS.
(C) DTL.
(D) RTL.ANS:b